EXPERIMENT 3

DATE:

Determination and plotting of Force of attraction between two electric Charges and two parallel  conductors separated by varying distance.

Aim:

To determine and plot the Force of attraction between two electric Charges and two parallel =conductors  separated by varying distance between them using MATLAB.

Software required:

•MATLAB (R2018 or later)

Theory:

Program:

Test case (i) Determination and plotting of force between two electric charges with varying distance

clc;

clear;

q1 = input('Enter the charge q1=');

q2 = input('Enter the charge q2=');

epsilon = 8.854e-12;

k = 1/(4*pi*epsilon);

r = linspace(0.1,10,100);

F = k*q1*q2./(r.^2);

disp('Force at r = 1 m is:');

disp(k*q1*q2/(1^2));

plot(r,F,'LineWidth',2);

xlabel('Distance (m)');

ylabel('Force (N)');

title('Force between two electric charges');

Output:-

Enter the charge q1=2e-6

Enter the charge q2=3e-6

Force at r = 1 m is:

0.0539 N

Graph: Force decreases with distance (1/r² behavior)

Test case (ii) Determination and plotting of force between two electric charges with different  magnitudes

Code:

clc;

q1 = 5e-6;

q2 = 2e-6;

epsilon = 8.854e-12;

k = 1/(4*pi*epsilon);

r = linspace(0.5,5,100);

F = k*q1*q2./(r.^2);

plot(r,F,'LineWidth',2);

xlabel('Distance (m)');

ylabel('Force (N)');

title('Force between two charges (different magnitudes)');

Output:-

• Force magnitude increases with increase in charge

• Inverse square decay maintained

Test case (iii) Determination and plotting of force per unit length between two parallel  conductors with varying distance

Code:

clc;

mu0 = 4*pi*1e-7;

I1 = 5;

I2 = 10;

d = linspace(0.1,10,100);

F = (mu0*I1*I2)./(2*pi*d);

plot(d,F,'LineWidth',2);

xlabel('Distance (m)');

ylabel('Force per unit length (N/m)');

title('Force between two parallel  conductors');

Output:

• Force decreases with increase in distance

• Linear inverse relation (1/d)

Test case (iv) Determination and plotting of force between two parallel conductors carrying  equal currents

Code:

clc;

mu0 = 4*pi*1e-7;

I1 = 3;

I2 = 3;

d = linspace(0.1,2,100);

F = (mu0*I1*I2)./(2*pi*d);

plot(d,F,'LineWidth',2);

xlabel('Distance (m)');

ylabel('Force per unit length (N/m)');

title('Force between conductors

(equal currents)');

Output:-

Force magnitude reduced compared to

Test case (iii)

Same inverse relationship

Results and Observations:

| Parameters q1=2e-6, q2=3e-6, r=1 m F = 0.0539 N q1=5e-6, q2=2e-6, r=1 m F = 0.0899 N I1=5 A, I2=10 A, d=1 m I1=3 A, I2=3 A, d=1 m | Observed/Simulated Result Theoretical Result Error Difference |  | ΔF ≈ 0 ΔF ≈ 0 ΔF ≈ 0 ΔF ≈ 0 |
| --- | --- | --- | --- |
|  |  | F = 0.0539 N |  |
|  |  | F = 0.0899 N |  |
|  | F = 1×10⁻⁵ N/m | F = 1×10⁻⁵ N/m |  |
|  | F = 1.8×10⁻⁶ N/m | F = 1.8×10⁻⁶ N/m |  |

Result :

Thus the program was verified for the force of attraction or repulsion acting along a straight line  between two electric charges is directly proportional to the product of the charges and inversely to the square  of the distance between them using SCILAB was successfully.